dnsop                                                             Momoka

Internet-Draft                                              WIDE Project

Obsoletes: 3901 (if approved)                                  T. Fiebig

Intended status: Best Current Practice                           MPI-INF

Expires: 4 June 2026                                     1 December 2025

DNS IPv6 Transport Operational Guidelines

draft-ietf-dnsop-3901bis-08

Abstract

This memo provides guidelines and documents Best Current Practice for

operating authoritative DNS servers as well as recursive and stub DNS

resolvers, given that queries and responses are carried in a mixed

environment of IPv4 and IPv6 networks.  This document recommends that

authoritative DNS servers as well as recursive DNS resolvers support

both IPv4 and IPv6.  It furthermore provides guidance for how

recursive DNS resolvers should select upstream DNS servers, if

both native and IPv4-embedded  IPv6 addresses are available.

This document obsoletes RFC 3901.

Discussion Venues

This note is to be removed before publishing as an RFC.

Source for this draft and an issue tracker can be found at

https://github.com/ietf-wg-dnsop/draft-ietf-dnsop-3901bis.

Status of This Memo

This Internet-Draft is submitted in full conformance with the

provisions of BCP 78 and BCP 79.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF).  Note that other groups may also distribute

working documents as Internet-Drafts.  The list of current Internet-

Drafts is at https://datatracker.ietf.org/drafts/current/.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

This Internet-Draft will expire on 4 June 2026.

Copyright Notice

Copyright (c) 2025 IETF Trust and the persons identified as the

document authors.  All rights reserved.

This document is subject to BCP 78 and the IETF Trust's Legal

Provisions Relating to IETF Documents (https://trustee.ietf.org/

license-info) in effect on the date of publication of this document.

Please review these documents carefully, as they describe your rights

and restrictions with respect to this document.  Code Components

extracted from this document must include Revised BSD License text as

described in Section 4.e of the Trust Legal Provisions and are

provided without warranty as described in the Revised BSD License.

Table of Contents

1.  Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   2

1.1.  Requirements Language . . . . . . . . . . . . . . . . . .   3

2.  Terminology . . . . . . . . . . . . . . . . . . . . . . . . .   3

3.  Name Space Fragmentation  . . . . . . . . . . . . . . . . . .   4

3.1.  Misconfigurations Causing IP Version Related Name Space

Fragmentation . . . . . . . . . . . . . . . . . . . . . .   4

3.2.  Network Conditions Causing IP Version Related Name Space

Fragmentation . . . . . . . . . . . . . . . . . . . . . .   6

3.3.  Reasons for Intentional IP Version Related Name Space

Fragmentation . . . . . . . . . . . . . . . . . . . . . .   8

4.  Policy Based Avoidance of Name Space Fragmentation  . . . . .   8

4.1.  Guidelines for Authoritative DNS Server Configuration . .   9

4.2.  Guidelines for Recursive DNS Resolvers  . . . . . . . . .  10

4.3.  Guidelines for DNS Stub Resolvers . . . . . . . . . . . .  10

5.  Security Considerations . . . . . . . . . . . . . . . . . . .  11

6.  IANA Considerations . . . . . . . . . . . . . . . . . . . . .  11

Acknowledgments . . . . . . . . . . . . . . . . . . . . . . . . .  11

References  . . . . . . . . . . . . . . . . . . . . . . . . . . .  11

Normative References  . . . . . . . . . . . . . . . . . . . . .  11

Informative References  . . . . . . . . . . . . . . . . . . . .  14

Authors' Addresses  . . . . . . . . . . . . . . . . . . . . . . .  15

1.  Introduction

Despite IPv6 being first discussed since the mid-1990s [RFC2460],

consistent deployment throughout the whole Internet has not yet been

accomplished [RFC9386].  Hence, the Internet still consists of

IPv4-only, dual-stack (networks supporting both IP versions), and

IPv6-only networks.

This creates a complex landscape where authoritative DNS servers

might be accessible only via specific network protocols

[V6DNSRDY-23].  At the same time, DNS resolvers may only be able to

access the Internet via either IPv4 or IPv6 connectivity.  This poses a challenge

for such resolvers because they may receive queries for names that

have authoritative DNS servers which do not support the same IP

version as the resolver.

[RFC3901] was initially written at a time when IPv6 deployment was

not widespread, focusing primarily on maintaining name space

continuity within the IPv4 landscape.  Two decades later, IPv6 is not

only widely deployed but also becoming the de facto standard in many

areas (mobile networks, data centers, etc.).  This document expands the scope of [RFC3901] by

recommending IPv6 connectivity for authoritative DNS servers, as well

as recursive and stub DNS resolvers.

This document provides:

*  Guidance on IP version related name space fragmentation and best practices for

avoiding it.

*  Guidelines for configuring authoritative DNS servers for zones.

*  Guidelines for operating recursive DNS resolvers.

*  Guidelines for stub DNS resolvers.

While transition and co-existence setups may mitigate

some of the DNS resolution issues in a mixed protocol-version

Internet, making DNS data accessible over both IPv4 and IPv6 is the

most robust and flexible approach.  This approach allows resolvers to

retrieve the information they need without requiring intermediary

translation or encapsulation services which may introduce additional

failure cases.

1.1.  Requirements Language

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "NOT RECOMMENDED", "MAY", and

"OPTIONAL" in this document are to be interpreted as described in BCP

14 [RFC2119] [RFC8174] when, and only when, they appear in all

capitals, as shown here.

2.  Terminology

This document uses DNS terminology as described in [RFC9499].

Furthermore, the following terms are used with a defined meaning:

IPv4 name server:

A name server providing DNS services reachable via IPv4.  It does

not imply anything about what DNS data is served, but means that

the name server receives and answers queries over IPv4.

IPv6 name server:

A name server providing DNS services reachable via IPv6.  It does

not imply anything about what DNS data is served, but means that

the name server receives and answers queries over IPv6.

Dual-stack name server:

A name server that is both an "IPv4 name server" and an "IPv6

name server".

3.  Name Space Fragmentation

A resolver that tries to look up a name starts out at the root, and

follows referrals until it is referred to a name server set that is

authoritative for the name.  If it is referred to a name server set

that is, based on a referral, only contains name servers that are

exclusively reachable via an IP address family that the resolver does not

support, the resolver is unable to continue DNS resolution.

If this occurs, the DNS has, effectively, fragmented based on the

recursive DNS resolver's and authoritative DNS server's mismatching

IP version support.

With the deployment of both IPv4 and IPv6, name space fragmentation can occur for

different reasons.  One reason is that DNS zones are consistently

configured to support only either IPv4 or IPv6.  Another reason is

due to misconfigurations that make a zone unresolvable by either IPv4-only

or IPv6-only resolvers.  The latter cases are often hard to identify,

as the impact of misconfigurations for only one IP version (IPv4 or

IPv6) may be hidden in a dual-stack setting.  In the worst case, a

specific name may only be resolvable via dual-stack enabled

resolvers.

3.1.  Misconfigurations Causing IP Version Related Name Space

Fragmentation

Even when an administrator assumes that they have enabled support for

a specific IP version on their authoritative DNS server, various

misconfigurations may break the DNS delegation chain of a zone for

that protocol version and prevent any of its records from resolving for

clients only supporting that IP version.  These misconfigurations can

be kept hidden if most clients can successfully fall back to the

other IP version.

The following name related misconfigurations can cause broken

delegation for one IP version:

No A/AAAA records for NS names:

If all of the NS resource records (RR) for a zone in their parent zone have

either only A RRs or only AAAA RRs, then resolution via

the other IP version is not possible.

Missing glue:

If the name from an NS record for a zone is in-domain(i.e., the

name is within the zone or below), a parent zone needs to contain

both IPv4 and IPv6 glue records.  A parent needs to serve the

corresponding A and AAAA RRs in the additional section when returning

the NS RRs as the referral response [RFC9471].

No A/AAAA RR for in-domain NS:

If the parent provides glue records for both IP versions but the

child zone itself lacks corresponding A or AAAA RRs for its

in-domain name server names, resolution via the missing IP version

will fail during delegation revalidation (see, e.g.,

[I-D.ietf-dnsop-ns-revalidation]).

Zone of sibling domain NSes not resolving:

If the name from an NS record for a zone is sibling domain, the

corresponding zone needs to be resolvable via the IP version in

question as well.  It is insufficient if the name pointed to by

the NS record has an associated A or AAAA record correspondingly.

Parent zone not resolvable via one IP version:

For a zone to be resolvable via an IP version, the parent zones up

to the root zone needs to be resolvable via that IP version as

well.  Any zone not resolvable via the concerned IP version breaks

the delegation chain for all its children.

The above misconfigurations are not mutually exclusive.

Furthermore, any of the misconfigurations above may not only

materialize via a missing RR but also via an RR

providing the IP address of a name server that is not configured to

answer queries via that IP version [V6DNSRDY-23].

3.2.  Network Conditions Causing IP Version Related Name Space

Fragmentation

In addition to explicit misconfigurations in the served DNS zones,

network conditions may also influence a resolver's ability to resolve

names in a zone.  The most common issue are packets requiring

fragmentation given a reduced path MTU (PMTU) and MTU discards,

i.e., packets being dropped on-path due to exceeding the MTU of the

link to the next-hop without the sender being notified.  This can

manifest in the following ways:

DNS-over-UDP packets requiring fragmentation

When using EDNS(0) to communicate support for DNS messages larger

than 512 octets [RFC6891] via conventional DNS-over-UDP transport

according to [RFC1035], an IP packet carrying a DNS

response may exceed the PMTU for the path to a resolver.  If an

authoritative DNS server does not follow [RFC9715] (i.e., honors

EDNS(0) sizes larger than 1232 octets), it will try to fragment the

packet according to the discovered PMTU.  Such packets mostly

occur for DNSKEY responses with DNSSEC [RFC4034].

In general, DNS servers SHOULD follow [RFC9715], which

provides additional guidance on preventing fragmentation by

ensuring that the maximum DNS/UDP payload size does not exceed

1400 octets.  This can be accomplished by setting a corresponding

EDNS(0) size, with most implementations using a lower EDNS(0) size

of 1232 octets following [DNSFlagDay2020], to ensure that

generated packets always fit into lower bound of the IPv6 MTU of

1280, as defined in [RFC8200].  Hence, DNS servers MAY opt to set

an EDNS(0) size of 1232 octets following [DNSFlagDay2020].

Additionally, DNS servers MAY opt to explicitly not rely on path

MTU discovery [RFC4821] or PLPMTUD [RFC8899], by instead using

IPV6_USE_MIN_MTU=1 from [RFC3542] to avoid the need to

perform PMTU discovery.

DNS-over-TCP packets requiring fragmentation

A resolver can for various reasons also initiate connections via

TCP for resolution to an authoritative server.  However, similar

to the case of DNS-over-UDP, DNS-over-TCP may encounter MTU

discards, especially on IPv6, if PMTUD does not work, if the MSS

honored by the authoritative DNS server leads to IP packets

exceeding the PMTU.  In that case, similar to the case of DNS-

over-UDP, DNS resolution will time out when the recursive DNS

resolver did not receive a response in time.

[RFC9715] does not provide explicit guidance on mitigating this

issue.  However, transferring the guidance from [RFC9715], setting

an MSS of 1388 octets would reduce the impact of this issue.

Hence, DNS servers MAY set an MSS of no more than 1388 octets for

TCP connections.  Similarly, aligned with the recommendations of

the [DNSFlagDay2020], DNS servers MAY ensure that a total packet

size of 1280 octets is not exceeded by setting an MSS of 1220

octets.  Additionally, DNS servers MAY opt to set

IPV6_USE_MIN_MTU=1 from [RFC3542].

Broken IP Connectivity at the Resolver

Similar to authoritative servers, (stub) recursive resolvers may

face broken IP connectivity for either IPv4 or IPv6:

IPv4 connectivity for a DNS resolver may experience issues, e.g.,

if the resolver is deployed behind a Carrier Grade NAT (CGN) [RFC6888]

that implements strict timeouts on active sessions, or

limits the number of available port numbers for connections.  Similarly,

[RFC1918] addressing may be in use on the resolver, while address

translation is not performed, or, similar to the case for IPv6,

when the DNS resolver has a global IPv4 address, but that address

is not routed on the resolver's network.

IPv6 connectivity for a DNS resolver may experience issues, if,

e.g., a client has been assigned a global unicast IPv6 address,

but IPv6 traffic is not forwarded on the resolver's network.

Similarly, IPv6 connectivity can experience issues when IPv4-IPv6

transition technologies, e.g., NAT64 [RFC6146] on IPv6-mostly

networks [RFC9313], or NAT64 connectivity discovered through

PREF64 [RFC8781] or DNS64 [RFC7050] on IPv6-only networks are in

use.  There, the synthesized IPv6 addresses used in 464XLAT

[RFC6877] encounter additional PMTU fluctuation due to the

difference in header size between IPv4 and IPv6, possibly

impacting DNS resolution.

Note: This document only explicitly discusses DNS-

over-TCP and DNS-over-UDP.  However, several other transport methods

between recursive and authoritative DNS severs exist, including DNS

over various encrypted transports.  Some of these technologies

provide additional mechanisms for preventing the impact of a reduced

PMTU or MTU discards.  Guidance in this document focuses on IP

version support, and questions of the underlying transport protocol

(TCP or UDP).  If DNS servers use an additional protocol layer, e.g.,

DNS-over-TLS [RFC7858] or DNS-over-QUIC [RFC9250], for their

communication, and that protocol supports additional measures to

prevent fragmentation on the IP layer related issues, these measures

SHOULD be used for the connection.  Otherwise, if the protocol is not

resilient to IP layer fragmentation related issues by default, the

above guidance for TCP and UDP based connections SHOULD be applied

analogously.

3.3.  Reasons for Intentional IP Version Related Name Space

Fragmentation

Intentional IP related name space fragmentation occurs if an operator

consciously decides not to deploy IPv4 or IPv6 for a part of the

resolution chain.  Most commonly, this is realized by intentionally

not listing A/AAAA RRs for NS names.  At the time of writing, the

share of zones not resolvable via IPv4 is negligible, while a little

less than 40% of zones are not resolvable via IPv6 [V6DNSRDY-23].

However, as IPv4 address exhaustion progresses, IPv6 adoption is expected to

increase.

4.  Policy Based Avoidance of Name Space Fragmentation

With the final exhaustion of IPv4 address pools in RIRs, e.g., [RIPEV4], and

the progressing deployment of IPv6, IPv4 and IPv6 have become

comparably relevant.  Yet, while it is observed that the first zones

becoming exclusively IPv6 resolvable, there is still a major

portion of zones solely relying on IPv4 [V6DNSRDY-23].  Hence,

dual stack connectivity is still instrumental to be able to resolve

zones and avoid name space fragmentation.

Having zones served only by name servers reachable via one IP version

would fragment the DNS.  Hence, the need for a way to avoid this

fragmentation.

The recommended approach to maintain name space continuity is to use

administrative policies, as described in this section.

4.1.  Guidelines for Authoritative DNS Server Configuration

It is usually recommended that DNS zones contain at least two name

servers, which are geographically diverse and operate under different

routing policies [IANANS].  To reduce the chance of DNS name space

fragmentation, it is RECOMMENDED that at least two name servers for a

zone are dual-stack name servers.  Specifically, this means that the

following minimal requirements SHOULD be implemented for a zone:

IPv4 adoption:

Every DNS zone SHOULD be served by at least one IPv4-reachable

authoritative DNS server to maintain name space continuity.  The

delegation configuration (Resolution of the parent, resolution of

sibling domain names, glue) MUST NOT rely on IPv6 connectivity

being available.  Given the IPv4 address scarcity, operators MAY

opt not to provide DNS services via IPv4, if they can ensure that

all clients expected to resolve this zone do support DNS

resolution via IPv6.

IPv6 adoption:

Every DNS zone SHOULD be served by at least one IPv6-reachable

authoritative DNS server to maintain name space continuity.  To

avoid reachability issues, authoritative DNS servers SHOULD use

native IPv6 addresses instead of IPv4-converted IPv6 addresses

for receiving queries.  The

delegation configuration (Resolution of the parent, resolution of

sibling domain names, glue) MUST NOT rely on IPv4 connectivity

being available.

Consistency:

Both IPv4 and IPv6 transports SHOULD serve identical DNS data to

ensure a consistent resolution experience across different network

types.

Avoiding IP Fragmentation:

IP fragmentation has been reported to be fragile [RFC8900].

Furthermore, IPv6 transition technologies can introduce unexpected

MTU breaks (e.g., when NAT64 is used (Section 7 of [RFC7269])).  Therefore, IP

fragmentation SHOULD be avoided by following guidance on maximum

DNS payload sizes [RFC9715] and providing TCP fall back options

[RFC7766].  Furthermore, similar to the guidance in [RFC9715],

authoritative DNS servers MAY set an MSS of either 1388 (analogous

to [RFC9715]) or 1220 (analogous to the [DNSFlagDay2020]

suggestions) in TCP sessions carrying DNS responses.

To prevent name space fragmentation, zone validation processes SHOULD

ensure that:

*  There is at least one IPv4 address record and one IPv6 address

record available for the name servers of any child delegation

within the zone.

*  The zone's authoritative servers follow [RFC9715] for avoiding

fragmentation on DNS-over-UDP.

*  The zone's authoritative servers support DNS-over-TCP [RFC9210].

*  The zone's authoritative servers can be reached via IPv4 and IPv6

when performing DNS resolution via IPv4-only and IPv6-only

networks respectively.

4.2.  Guidelines for Recursive DNS Resolvers

Every recursive DNS resolver SHOULD be dual-stack.

While the zones that IPv6-only recursive DNS resolvers can resolve

are growing, they do not yet cover all zones.  Hence, a recursive DNS

resolver MAY be IPv6-only, if it uses a transition mechanism that

allows it to also query IPv4-only authoritative DNS servers, or uses

a configuration where it forwards queries failing IPv6-only DNS

resolution to a recursive DNS resolver that is able to perform DNS

resolution over IPv4.  For example, if a recursive DNS resolver is

aware of a PREF64 to use for NAT64 [RFC6146], either through static

configuration or by discovering it (e.g., [RFC8781]), it may synthesize IPv6

addresses for remote authoritative DNS servers.

Similarly, a recursive DNS resolver MAY be IPv4-only, if it uses a

configuration where such resolvers forward queries failing IPv4-only

DNS resolution to a recursive DNS resolver that is able to perform

DNS resolution over IPv6.

Finally, when responding to recursive queries sent by stub DNS

resolvers, a DNS resolver SHOULD follow the above guidance on

fragmentation avoidance (Section XXX) for communication

between authoritative DNS servers and recursive DNS resolvers

analogously.

4.3.  Guidelines for DNS Stub Resolvers

Contrary to authoritative DNS servers and recursive DNS resolvers,

stub DNS resolvers are more likely to find themselves in either an

IPv6-mostly or IPv4-only environment, as they are usually run on end-

hosts / clients.  Furthermore, a stub DNS resolver has to rely on

recursive DNS servers discovered for the local network, e.g., using

DHCPv4 [RFC3456], DHCPv6 [RFC8415], and/or SLAAC [RFC4862].  In that

case, the stub resolver may obtain multiple different IPv4 and IPv6

DNS resolver addresses to use.

To prioritize different IPv4 and IPv6 DNS resolver addresses, a stub

resolver SHOULD follow [RFC6724].  However, a stub DNS resolver

SHOULD NOT utilize IPv4-embedded IPv6 addresses if it is able to identify

them as such, e.g., by having discovered the PREF64 in use for the

network [RFC8781].

When providing multiple DNS servers to stub resolvers,

operators SHOULD consider that various implementations can only

configure a small set of possible DNS resolvers, e.g., only up to

three for libc, and additional resolvers provided may be ignored by

clients.

5.  Security Considerations

The guidelines described in this memo introduce no new security

considerations into the DNS protocol.

Recommendations for recursive and stub resolvers rely on a correctly

discovered PREF64.  Security issues may materialize if an incorrect

PREF64 is used.  Hence, guidance from [RFC9872] on securely

discovering PREF64 SHOULD be followed.

6.  IANA Considerations

This document requests IANA to update its technical requirements for

authoritative DNS servers to require both IPv4 and IPv6 addresses for

each authoritative server [IANANS].

Acknowledgments

Valuable input for this draft was provided by: Bob Harold, Andreas

Schulze, Tommy Jensen, Nick Buraglio, Jen Linkova, Tim Chown, Brian E

Carpenter, Tom Petch, Philipp S.  Tiesel, Mark Andrews, Stefan

Ubbink, Joe Abley, Gorry Fairhurst, Paul Vixie, Lorenzo Colitti,

David Farmer, Pieter Lexis, Ralf Weber, Philip Homburg, Marco Davids

Thank you for reading this draft.

References

Normative References

[RFC1035]  Mockapetris, P., "Domain names - implementation and

specification", STD 13, RFC 1035, DOI 10.17487/RFC1035,

November 1987, <https://www.rfc-editor.org/info/rfc1035>.

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119,

DOI 10.17487/RFC2119, March 1997,

<https://www.rfc-editor.org/info/rfc2119>.

[RFC2460]  Deering, S. and R. Hinden, "Internet Protocol, Version 6

(IPv6) Specification", RFC 2460, DOI 10.17487/RFC2460,

December 1998, <https://www.rfc-editor.org/info/rfc2460>.

[RFC3456]  Patel, B., Aboba, B., Kelly, S., and V. Gupta, "Dynamic

Host Configuration Protocol (DHCPv4) Configuration of

IPsec Tunnel Mode", RFC 3456, DOI 10.17487/RFC3456,

January 2003, <https://www.rfc-editor.org/info/rfc3456>.

[RFC3542]  Stevens, W., Thomas, M., Nordmark, E., and T. Jinmei,

"Advanced Sockets Application Program Interface (API) for

IPv6", RFC 3542, DOI 10.17487/RFC3542, May 2003,

<https://www.rfc-editor.org/info/rfc3542>.

[RFC3901]  Durand, A. and J. Ihren, "DNS IPv6 Transport Operational

Guidelines", BCP 91, RFC 3901, DOI 10.17487/RFC3901,

September 2004, <https://www.rfc-editor.org/info/rfc3901>.

[RFC4034]  Arends, R., Austein, R., Larson, M., Massey, D., and S.

Rose, "Resource Records for the DNS Security Extensions",

RFC 4034, DOI 10.17487/RFC4034, March 2005,

<https://www.rfc-editor.org/info/rfc4034>.

[RFC4821]  Mathis, M. and J. Heffner, "Packetization Layer Path MTU

Discovery", RFC 4821, DOI 10.17487/RFC4821, March 2007,

<https://www.rfc-editor.org/info/rfc4821>.

[RFC4862]  Thomson, S., Narten, T., and T. Jinmei, "IPv6 Stateless

Address Autoconfiguration", RFC 4862,

DOI 10.17487/RFC4862, September 2007,

<https://www.rfc-editor.org/info/rfc4862>.

[RFC6146]  Bagnulo, M., Matthews, P., and I. van Beijnum, "Stateful

NAT64: Network Address and Protocol Translation from IPv6

Clients to IPv4 Servers", RFC 6146, DOI 10.17487/RFC6146,

April 2011, <https://www.rfc-editor.org/info/rfc6146>.

[RFC6724]  Thaler, D., Ed., Draves, R., Matsumoto, A., and T. Chown,

"Default Address Selection for Internet Protocol Version 6

(IPv6)", RFC 6724, DOI 10.17487/RFC6724, September 2012,

<https://www.rfc-editor.org/info/rfc6724>.

[RFC6888]  Perreault, S., Ed., Yamagata, I., Miyakawa, S., Nakagawa,

A., and H. Ashida, "Common Requirements for Carrier-Grade

NATs (CGNs)", BCP 127, RFC 6888, DOI 10.17487/RFC6888,

April 2013, <https://www.rfc-editor.org/info/rfc6888>.

[RFC6891]  Damas, J., Graff, M., and P. Vixie, "Extension Mechanisms

for DNS (EDNS(0))", STD 75, RFC 6891,

DOI 10.17487/RFC6891, April 2013,

<https://www.rfc-editor.org/info/rfc6891>.

[RFC7766]  Dickinson, J., Dickinson, S., Bellis, R., Mankin, A., and

D. Wessels, "DNS Transport over TCP - Implementation

Requirements", RFC 7766, DOI 10.17487/RFC7766, March 2016,

<https://www.rfc-editor.org/info/rfc7766>.

[RFC7858]  Hu, Z., Zhu, L., Heidemann, J., Mankin, A., Wessels, D.,

and P. Hoffman, "Specification for DNS over Transport

Layer Security (TLS)", RFC 7858, DOI 10.17487/RFC7858, May

2016, <https://www.rfc-editor.org/info/rfc7858>.

[RFC8174]  Leiba, B., "Ambiguity of Uppercase vs Lowercase in RFC

2119 Key Words", BCP 14, RFC 8174, DOI 10.17487/RFC8174,

May 2017, <https://www.rfc-editor.org/info/rfc8174>.

[RFC8200]  Deering, S. and R. Hinden, "Internet Protocol, Version 6

(IPv6) Specification", STD 86, RFC 8200,

DOI 10.17487/RFC8200, July 2017,

<https://www.rfc-editor.org/info/rfc8200>.

[RFC8415]  Mrugalski, T., Siodelski, M., Volz, B., Yourtchenko, A.,

Richardson, M., Jiang, S., Lemon, T., and T. Winters,

"Dynamic Host Configuration Protocol for IPv6 (DHCPv6)",

RFC 8415, DOI 10.17487/RFC8415, November 2018,

<https://www.rfc-editor.org/info/rfc8415>.

[RFC8899]  Fairhurst, G., Jones, T., Tüxen, M., Rüngeler, I., and T.

Völker, "Packetization Layer Path MTU Discovery for

Datagram Transports", RFC 8899, DOI 10.17487/RFC8899,

September 2020, <https://www.rfc-editor.org/info/rfc8899>.

[RFC9210]  Kristoff, J. and D. Wessels, "DNS Transport over TCP -

Operational Requirements", BCP 235, RFC 9210,

DOI 10.17487/RFC9210, March 2022,

<https://www.rfc-editor.org/info/rfc9210>.

[RFC9250]  Huitema, C., Dickinson, S., and A. Mankin, "DNS over

Dedicated QUIC Connections", RFC 9250,

DOI 10.17487/RFC9250, May 2022,

<https://www.rfc-editor.org/info/rfc9250>.

[RFC9471]  Andrews, M., Huque, S., Wouters, P., and D. Wessels, "DNS

Glue Requirements in Referral Responses", RFC 9471,

DOI 10.17487/RFC9471, September 2023,

<https://www.rfc-editor.org/info/rfc9471>.

Informative References

[DNSFlagDay2020]

"DNS flag day 2020", <https://dnsflagday.net/2020/>.

[I-D.ietf-dnsop-ns-revalidation]

Huque, S., Vixie, P. A., and W. Toorop, "Delegation

Revalidation by DNS Resolvers", Work in Progress,

Internet-Draft, draft-ietf-dnsop-ns-revalidation-11, 19

October 2025, <https://datatracker.ietf.org/doc/html/

draft-ietf-dnsop-ns-revalidation-11>.

[IANANS]   IANA, "Technical requirements for authoritative name

servers",

<https://www.iana.org/help/nameserver-requirements>.

[RFC1918]  Rekhter, Y., Moskowitz, B., Karrenberg, D., de Groot, G.

J., and E. Lear, "Address Allocation for Private

Internets", BCP 5, RFC 1918, DOI 10.17487/RFC1918,

February 1996, <https://www.rfc-editor.org/info/rfc1918>.

[RFC6877]  Mawatari, M., Kawashima, M., and C. Byrne, "464XLAT:

Combination of Stateful and Stateless Translation",

RFC 6877, DOI 10.17487/RFC6877, April 2013,

<https://www.rfc-editor.org/info/rfc6877>.

[RFC7050]  Savolainen, T., Korhonen, J., and D. Wing, "Discovery of

the IPv6 Prefix Used for IPv6 Address Synthesis",

RFC 7050, DOI 10.17487/RFC7050, November 2013,

<https://www.rfc-editor.org/info/rfc7050>.

[RFC7269]  Chen, G., Cao, Z., Xie, C., and D. Binet, "NAT64

Deployment Options and Experience", RFC 7269,

DOI 10.17487/RFC7269, June 2014,

<https://www.rfc-editor.org/info/rfc7269>.

[RFC8781]  Colitti, L. and J. Linkova, "Discovering PREF64 in Router

Advertisements", RFC 8781, DOI 10.17487/RFC8781, April

2020, <https://www.rfc-editor.org/info/rfc8781>.

[RFC8900]  Bonica, R., Baker, F., Huston, G., Hinden, R., Troan, O.,

and F. Gont, "IP Fragmentation Considered Fragile",

BCP 230, RFC 8900, DOI 10.17487/RFC8900, September 2020,

<https://www.rfc-editor.org/info/rfc8900>.

[RFC9313]  Lencse, G., Palet Martinez, J., Howard, L., Patterson, R.,

and I. Farrer, "Pros and Cons of IPv6 Transition

Technologies for IPv4-as-a-Service (IPv4aaS)", RFC 9313,

DOI 10.17487/RFC9313, October 2022,

<https://www.rfc-editor.org/info/rfc9313>.

[RFC9386]  Fioccola, G., Volpato, P., Palet Martinez, J., Mishra, G.,

and C. Xie, "IPv6 Deployment Status", RFC 9386,

DOI 10.17487/RFC9386, April 2023,

<https://www.rfc-editor.org/info/rfc9386>.

[RFC9499]  Hoffman, P. and K. Fujiwara, "DNS Terminology", BCP 219,

RFC 9499, DOI 10.17487/RFC9499, March 2024,

<https://www.rfc-editor.org/info/rfc9499>.

[RFC9715]  Fujiwara, K. and P. Vixie, "IP Fragmentation Avoidance in

DNS over UDP", RFC 9715, DOI 10.17487/RFC9715, January

2025, <https://www.rfc-editor.org/info/rfc9715>.

[RFC9872]  Buraglio, N., Jensen, T., and J. Linkova, "Recommendations

for Discovering IPv6 Prefix Used for IPv6 Address

Synthesis", RFC 9872, DOI 10.17487/RFC9872, September

2025, <https://www.rfc-editor.org/info/rfc9872>.

[RIPEV4]   RIPE NCC, "The RIPE NCC has run out of IPv4 Addresses",

November 2019, <https://www.ripe.net/publications/news/

about-ripe-ncc-and-ripe/the-ripe-ncc-has-run-out-of-

ipv4-addresses>.

[V6DNSRDY-23]

Streibelt, F., Sattler, P., Lichtblau, F., Hernandez-

Gañán, C., Gasser, O., and T. Fiebig, "How Ready is DNS

for an IPv6-Only World?", March 2023,

<https://link.springer.com/

chapter/10.1007/978-3-031-28486-1_22>.

Authors' Addresses

Momoka Yamamoto

WIDE Project

Email: momoka.my6@gmail.com

Tobias Fiebig

Max-Planck-Institut fuer Informatik

Campus E14

66123 Saarbruecken

Germany

Phone: +49 681 9325 3527

Email: tfiebig@mpi-inf.mpg.de